3DLE Command Structure

Mar 7, 2022

UART/Serial Port Emulation over BLE

This will be done using a GATT Server and GAT Client. The 3DLE unit will be the GATT Server and the 3DLE app will be the GATT Client.

This will have one service. The 128-bit Service UUID is 5E400001-B5A3-F393-E0A9-E50E24DCCA9E.

This service exposes two characteristics: one for transmitting and one for receiving.

- TX Characteristic (UUID: 5E400003-B5A3-F393-E0A9-E50E24DCCA9E)

  - Properties: NOTIFY, READ

  - Descriptors: Client Characteristic Configuration

    - UUID: 0x2902

- RX Characteristic (UUID: 5E400002-B5A3-F393-E0A9-E50E24DCCA9E)

  - Properties: WRITE

This video (https://drive.google.com/file/d/1ztfKEAnGFigIyUSkC9bWmrs3be0rOYP-/view?usp=sharing) demonstrates the GATT Server and GATT Client running on 2 android phones demonstrating the following functionalities:

- GATT Client (3DLE app) sending data to GATT Server (3DLE unit)

  - Enter Command: !B0A

  - GATT Client sends !B0A

  - GATT Server set R Mic Level to 0

  - GATT Client receives acknowledgement: 06

- GATT Server (3DLE unit) sends data to GATT Client (3DLE app)

  - Move L Mic Level on GATT Server to +6

  - GATT Server sends notification !C0D

  - GATT Client receives !C0D

- GATT Client (3DLE app) reads data from GATT Server (3DLE unit)

  - Enter Command:?C

  - GATT Client sends ?C

  - GATT Client receives ?C0D

3DLE App Command Screen:

UART Commands

Channel Gain

Note: The rough equivalent to the Monitor Input of the 3DME would here be the BT audio input.  We do not provide a level control for the Monitor Input of the 3DME.  As any playback app on the phone would have its level control, am I correct in assuming that our app providing level control over this input would be redundant and confusing, as would be passive earphones with their own level control?  But should there be a BT audio On/Off switch, to be able to turn off any unwanted sounds emanating from a connected phone?

Read current RIGHT channel gain:

- Send “?B”

- Receive “?B” + one byte $00 - $10

Write current RIGHT channel gain:

- Send “!Bxx”, where xx is the desired gain per below table.

- Receive acknowledgement  $06

Receive notification of current RIGHT channel gain change:

- Receive “?Bxx”, where xx is the desired gain per below table.

Read current LEFT channel gain:

- Send “?C”

- Receive “?C” + one byte $00 - $10

Write current LEFT channel gain

- Send “!Cxx”, where xx is the desired gain per below table.

- Receive acknowledgement  $06

Receive notification of current LEFT channel gain change:

- Receive “?Cxx”, where xx is the desired gain per below table.

$00 = OFF

$01 = -24dB

$02 = -19dB

$03 = -15dB

$04 = -12dB

$05 = -10dB

$06 = -8dB

$07 = -6dB

$08 = -4dB

$09 = -2dB

$0A =  0dB

$0B = +2dB

$0C = +4dB

$0D = +6dB

$0E = +8dB

$0F = +10dB

$10 = +12dB

Limiter Threshold

Note: The Limiter ON/OFF switch acts on both Left and Right, even when thresholds are separated, as does the L/R Limiter Action Unlink/Link switch.

Read current RIGHT channel limiter threshold:

- Send “?D”

- Receive “?D” + one byte $00 - $08

Write current RIGHT channel limiter threshold:

- Send “!Dxx”, where xx is the limiter threshold per below table.

- Receive acknowledgement  $06

Read current LEFT channel limiter threshold:

- Send “?E”

- Rreceive “?E” + one byte $00 - $08

Write current LEFT channel limiter threshold:

- Send “!Exx”, where xx is the limiter threshold per below table.

- Receive acknowledgement  $06

$00 = 76

$01 = 80

$02 = 84

$03 = 88

$04 = 92

$05 = 96

$06 = 100

$07 = 104

$08 = OFF

Limiter Link

Read current limiter link status:

- Send “?F”

- Receive “?F” + one byte $00 or $01  $01 = link enabled

Write current limiter link status:

- Send “!Fxx”, where xx is either $00 or $01 per above.

- Receive acknowledgement  $06

Equalizer

Band 1 (60 Hz)

Read current LEFT EQ band 1:

- Send “?G”

- Receive “?G” + one byte $00 to $0C

Write current LEFT EQ band1:

- Send “!Gxx”, where xx is the EQ value per below table.

- Receive acknowledgement  $06

Read current RIGHT EQ band 1:

- Send “?M”

- Receive ?M” + one byte $00 to $0C

Write current RIGHT EQ band1:

- Send “!Mxx”, where xx is the EQ value per below table.

- Receive acknowledgement  $06

$00 = -12dB

$01 = -10dB

$02 = -8dB

$03 = -6dB

$04 = -4dB

$05 = -2dB

$06 = 0dB

$07 = +2dB

$08 = +4dB

$09 = +6dB

$0A = +8dB

$0B = +10dB

$0C = +12dB

The following table is for Bands 2 – 7 (identical to Band 1 except for command letter)

Band 2 (140 Hz). 	Left EQ2 use “H” to read and write	Right EQ2 use “N” to read and write

Band 2 (330 Hz). 	Left EQ3 use “I” to read and write	Right EQ3 use “O” to read and write

Band 2 (770 Hz).	Left EQ4 use “J” to read and write	Right EQ4 use “P” to read and write

Band 2 (1800 Hz).	Left EQ5 use “K” to read and write	Right EQ5 use “Q” to read and write

Band 2 (4300 Hz).	Left EQ6 use “L” to read and write	Right EQ6 use “R” to read and write

Band 2 (10 kHz).	Left EQ7 use “S” to read and write	Right EQ7 use “T” to read and write

DSP Bypass

In the 3DME app, this is labeled EQ <-> Flat.  The Flat position bypasses the only digital signal processing (the EQ) in the product, which improves dynamic range a bit, eliminates the 0.3 msec of latency, and enables a quick A-B comparison of with EQ and without.  The desired results could be executed differently in the 3DLE.

The EQ section (the DSP section in the 3DME) can be set to flat.  There is an advantage to bypassing this processing in the Flat position in the 3DME (latency, dynamic range, remembering EQ settings), but if there is no advantage here, the controls can simply be set to 0 dB. If not bypassed (just set to zero), then the EQ settings should still be remembered, either in the hardware or the app, so that the previous settings can be returned to, enabling quick A/B comparison.

Read DSP bypass status:

- Send “?U”

- Receive “?U” + one byte $01 = OFF, $00 = ON

Write DSP bypass

- Send “!Uxx”, where $01 = OFF and $00 = ON

- Receive acknowledgement  $06.

Save default  gain values

Write save current gain values as default:

- Send “!V”

- Receive acknowledgement  $06

Mic Level Joining Status

Note: When the app screen is switched back to “Join” from “Separate”, the Left and Right Mic gains are both set to the lower of the previous Left or Right settings, both in “Step” mode and for both settings of  “Set” mode.

Note: When the app screen is switched back to “Join” from “Separate”, the Left and Right thresholds are both set to the lower of the previous Left or Right settings.

Read current gain joining status:

- Send “?W”

- Receive “?W” + one byte $00 (separated ) or $01  (joined)

Write current gain joining status:

- Send “!Wxx”, where xx is either 00 (separated ) or $01  (joined).

- Receive acknowledgement  $06

CROS (contralateral routing of signal)

Off, Left-To-Right, or Right-to-Left

Read CROS status:

- Send “?X”

- Receive “?X” + one byte (xx), where  xx is detailed in the below table.

Write CROS status:

- Send “!Xxx”, where  xx is detailed in the below table.

- Receive acknowledgement  $06

| xx | L to R | R to L |
| --- | --- | --- |
| $00 | OFF | OFF |
| $01 | ON | OFF |
| $02 | OFF | ON |

Gain Delta

If the left and right microphone gains are set differently, this is the offset to maintain as the pushbuttons step the gain up and down.  This control signal should not be necessary if the button action default is to step the left and right mic gains up and down an equal number of steps.

Read channel gain delta:

- Send “?Y”

- Receive  “?Y” + one byte = difference between left and right channel gain steps

Battery Status

Read Battery Status:

- Send “?a”

- Receive  “?a” + one byte (xx), where  xx is detailed in the below table.

$00 = FAST BLINK BATT 1,

$01 = SLOW BLINK BATT 1

$02 = BATT 1

$03 = BATT 1&2

$04 = BATT 1 & 2 & 3

$05 = BATT 1 & 2 & 3 & 4.

Serial Number

Read unit serial number:

- Send “?b”

- Receive “?b” + 3 bytes.  $00 $00 $00 (00000000) to $FF $FF $FF (16777215)

Write unit serial number:

- Send “!bxxyyzzMMYY” where xx yy zz range from $00 $00 $00 (00000000) to $FF $FF $FF (16777215) and MM = Month (1-12) and YY = Year (00-99).

- Receive acknowledgement  $06

Equalizer Joining Status

Read EQ  Joining status:

- Send “?c”

- Receive “?c” + one byte  $00 (separated)  or $01 (joined)

Write EQ  joining status:

- Send “!cXX”, where XX = $00 (separated) or $01 (joined).

- Receive acknowledgement  $06

Seal Test Control

Write Seal Test:

- Send “!dXX”, where XX = $00 (OFF) or $01 (ON).

- Receive acknowledgement  $06

Limiter Joining Status

Read limiter joining status:

- Send “?e”

- Receive “?e” + one byte  $00 (separated)  or $01 (joined)

Write limiter joining status:

- Send “!eXX”, where XX = $00 (separated) or $01 (joined).

- Receive acknowledgement  $06

Button Function

This controls whether the pushbuttons are in the “Step” mode or the “Set” mode, where the buttons switch between two pre-established levels (e.g. 0 dB and -15 dB)

The buttons can be operated either in Step Mode, where each button press steps the microphone level up or down one step at a time (the default mode; also note that the number of any offset steps between the two ears is maintained) or Preset Mode, where the buttons toggle between two settings of the microphone levels.  We are in the process of evaluating the addition of a third mode, which will be “Buttons Off”, which will prevent accidental changing of the mic levels.

Read button function status:

- Send “?f”

- Receive “?f” + one byte $00 (steps) or $01 (presets)

Write button function status:

- Send “!fXX”, where XX = $00 (steps) or  $01 (presets).

- Receive acknowledgement  $06

Left UP button preset

Read Left UP preset:

- Send “?g”

- Receive “?g” + one byte $00 - $10 per Left Volume definition above

Write Left UP preset:

- Send “!gXX”, where XX = Left Volume gain word per gain definition above.

- Receive acknowledgement  $06

Right UP button preset

Read Right UP preset:

- Send “?h”

- Receive “?h” + one byte $00 - $10 per Right Volume definition above

Write Left UP preset:

- Send “!gXX”, where XX = Right Volume gain word per gain definitions above.

- Receive acknowledgement  $06

Left DOWN button preset

Read Left UP preset:

- Send “?i”

- Receive “?i” + one byte $00 - $10 per Left Volume definition above

Write Left DOWN preset:

- Send “!iXX”, where XX = Left Volume gain word per gain definition above.

- Receive acknowledgement  $06

Right DOWN button preset

Read Right DOWN preset:

- Send “?j”

- Receive “?j” + one byte $00 - $10 per Right Volume definition above

Write Left DOWN preset:

- Send “!jXX”, where XX = Right Volume gain word per gain definition above.

- Receive acknowledgement  $06

Firmware Version

Read firmware version number:

- Send “?k”

- Receive “?k” + one byte. Number is 10X of the version.

Example: $1B = 27 or version 2.7

Load all EQ at once

Write 14 EQ registers:

- Send “!mEQL1” &  “EQL2”  &  “EQL3”  &  “EQL4”  &  “EQL5”  &  “EQL6”  &  “EQL7”  &  “EQR1”  &

“EQR2”  & “EQR3”  & “EQR4”  & “EQR5”  & “EQR6” & “EQR7”.

Example: “!m0102030405060701020304050607”

- Receive acknowledgement  $06

Set 3DME BTLE Power

Read BTLE Power:

- Send “?n”

- Receive “?n” + one byte (XX), where “XX” is the output power in dBm

Write BTLE Power:

- Send “!nXX”, where “XX” is the output power in dBm.

- Receive acknowledgement  $06

BT Audio Processing Bypass

An analogous Monitor Processing Bypass function is being added to the 3DME BT Gen2.  It enables the Monitor In signal to bypass the EQ and limiter to accommodate those “A-list” performers with their own monitor engineers who want to do it themselves without our interference.  Considering the BT audio to correspond to the Monitor audio, there doesn’t seem to be a similar  reason to include this option in the 3DLE.  It’s not shown in the V1.1 block diagram.  The following is included just to maintain the option.

Read BT Audio Processing Bypass status:

- Send “?p”

- Receive “?p” + one byte $00 (disabled) or $01 (enabled)

Write BT Audio Processing Bypass status:

- Send “!pXX”, where “XX” = $00 (disabled), $01 (enabled).

- Receive acknowledgement  $06

Hardware Version

Read firmware version number:

Send “?q”

Receive “?q” + one byte (XX), where “XX” is detailed in the below table.

$01 = V2

dBCheck

This function is not present in the 3DME.  It’s described in the V1.1 Block Diagram document, but adding bit more here, some of which Morgan and I have discussed.

The app needs to know the average power level over a time period, perhaps hours, to determine the “safe” listening times according to NIOSH and OSHA for the average power level over that time period.  The app collects the power level data from the four data streams (left and right of microphone and reproduced sound) and calculates the NIOSH and OSHA “safe” times for the ambient and reproduced sound from the accumulated power level data..  (The displayed “safe” times will be based on the worst of the left or right ears.)  The app will also display the 1-second average dBA-SPL levels.

When the app tells the hardware to start returning dBCheck information, the hardware will, for each of the four data streams, calculate the 1-second rms average power levels and send them at 1-second intervals by::

- Square the sampled, A-weighted linear level representations.

- Average these squared values over 1 second intervals..

- Take the square root of the 1-second averaged squared values..

- Return each of these four values each second to the app for further processing, until the app says to stop..

- .

See 3DLE dBCheck Functionality Programming Outline V1.docx, which shows earlier thinking, but is still mostly relevant:

https://docs.google.com/document/d/14DFr9uI1Y8L3Xl54wHt0yrzNaIKBXcgW/edit?usp=sharing&ouid=115703374070549883626&rtpof=true&sd=true