Instruction for BT upgrade, for V2.0

- Install BT programming software Media Product Tool V2.10.05 from https://drive.google.com/file/d/1sDIpBoRpTI0spnrso8B8LOXcQjUbzPOO/view?usp=sharing

- Please run the BT programming software “Launch.exe” in the PC.

- Select “FACTORY_INFORMATION_ERASE”, as shown.  (Ignore the warning.)

- Click the “Update” button, select the new BT firmware file for programming, and then click the "Replace" button。

- Connect the unit to PC with type C and turn the unit ON.

- Long press the MODE button to enter the BT programming mode.  (Note: For V1.5, quick press the “mode” button twice instead of one long press.)

Key function description:

“vol+”:

QUICK PRESS ONCE：  UP VOLUME

“vol-“:

QUICK PRESS ONCE：  DOWN VOLUME

“power”:

QUICK PRESS ONCE：  POWER ON

QUICK PRESS ONCE：  POWER OFF

“mode”

QUICK PRESS ONCE：  CHANGE MODE  ( BT, MIC, BT+MIC )

QUICK PRESS TWICE：  ENTER BT PROGRAMMING MODE

Note: For ASI V2.0 , long press the MODE button to enter BT download mode.

- Wait for the "Download" button to turn blue, as below.

- Click the blue "Download" button. When you see the following success prompt, it indicates that the new BT firmware has been upgraded successfully.

Appendix:  Key function description:

“vol+”:

QUICK PRESS ONCE：  UP VOLUME

“vol-“:

QUICK PRESS ONCE：  DOWN VOLUME

“power”:

QUICK PRESS ONCE：  POWER ON

QUICK PRESS ONCE：  POWER OFF

“mode”

ASI V2.0:  LONG PRESS:    ENTER BT PROGRAMMING MODE

ASI V1.5:  QUICK PRESS ONCE：  CHANGE MODE  ( BT, MIC, BT+MIC );

QUICK PRESS TWICE：  ENTER BT PROGRAMMING MODE

.