BATTERY SUPPLY TEST FOR SIU

SIU: 11663-A-00  (FW: v5013)

SD PLUGS : 11737-D-00

SENSOR:  12102-B-00

(All current test on battery side)

1, Plug in USB charging

-LED flashing white color.

-Charging current about 80mA.

-Will turn the SIU on when plug in USB in charging.

-Press button to turn the SIU off, The battery stop charging in the same time.

- Press button to turn the SIU off ,Can not turn the SIU on when press button again, Must re-connect the USB or just used battery alone can be turn SIU on again.

2, Battery Supply（with sensor）

- LED flashing white color.

- Current consumption about 2-15mA, BT pairing about 57mA.

- in Power off mode, leakage current about 35Ma in 100ms interval.

-Added 2K resistor between B1_BIN and B3_VDD of the fuel gauge, leakage current can be going down to 200-400uA.

- When the battery voltage going down to about 2.9V,The SIU will auto turn off for protect the battery.

3, Battery Supply（working in wireless mode）

- LED flashing green color.

- Current consumption about 25mA.