Supreme-On

Wear-Detection Test

Function

The headphone has an integrated sensor for detecting if it is put on the head or taken off. This sensor is required for the feature of automatic playback control, i.e. pausing the audio playback when headphone is taken off and resuming after being put up again.

The system should pause playback when BOTH sides are taken off (AND configuration) and should play again when BOTH sides are back ON (AND configuration).

Associated software/firmware supports timed functions for optimal overall user experience, e.g. implanting dead-out times, optimal logical combination of the two sensor outputs etc.

Requirements:

All following requirements need to be met under following conditions:

Temperature:

Humidity:

Skin condition:

- No triggering of pause when both sides are on the ears also when wearing the headphone for a long period of time (8 hours).

- No false triggering of pause when only one ear pad is touching the ear (left/right), also when fast putting the ear pad (right/left) on the ear and off the ear again in a time-frame of 4 seconds. Repeated 20 times.

- No false triggering of play/pause when fast putting the headphone on the head and off the head again in a time-frame of 4 seconds. Repeated 20 times.

- No false triggering of play when wearing the headphone around the neck.

- The perceived reaction time to trigger play/pause must be below 2 seconds.

- False triggering of play/pause when under the following conditions can be neglected:

  - in collapsed state

  - wearing a hat

  - Hair style

  - when wet with perspiration

| Firmware Version | MCU_V3.1,BT_V25 |
| --- | --- |

Pre-condition: Headphone powered off.

Task 1:

Turn on headphone. Put headphone (both sides) on the head & start playback. Listen to playback for 20 mins

| Number of wrong triggering | Zero |
| --- | --- |

Task 2:

Pre-condition: headphone (both sides) on the head and powered on, playback active.

- Take headphone off the head (BOTH SIDES) and wait 3s.

- Put the headphone back on the head (BOTH SIDES) and wait 3s.

- repeat 20 times.

| Number of wrong triggering | Sometimes first time take headphone off not into pause mode |
| --- | --- |
| Average Reaction time pause | 20 |
| Average Reaction time play | 20 |

Task 3:

Pre-condition: headphone (both sides) on the head and powered on, playback active.

- Take headphone off the head (BOTH SIDES) and wait 8s.

- Put the headphone back on the head (BOTH SIDES) and wait 8s.

- repeat 20 times.

| Number of wrong triggering | Sometimes first time take headphone off not into pause mode |
| --- | --- |
| Average Reaction time pause | 20 |
| Average Reaction time play | 20 |

Task 4:

Pre-condition: headphone (BOTH SIDES) on the head and powered on, playback active.

- Move one earcup behind ear and wait 4s. (needs to continue playback).

- Move the earcup onto ear again. (needs to continue playback).

Repeat 20 times with both Left and Right earcups

| Number of wrong triggering (Left) | zero |
| --- | --- |
| Number of wrong triggering (Right) | zero |

Task 6:

Pre-condition: headphone (both sides) on the head and powered on, playback active.

- Take headphone off the head and put it around the neck.

- Wear it around the neck and walk for 13 minutes.

- Put headphone back on the head.

| Number of wrong triggering | zero |
| --- | --- |
| Reaction time pause | 2 |
| Reaction time play | 2 |

Task 7:

Pre-condition: headphone (both sides) on the head and powered on, playback active.

- Move one earcup away from the ear for 2s.

- Quickly replace the earcup on the ear

- Put headphone back on the head.

| Number of wrong triggering | zero |
| --- | --- |
| Reaction time pause | 20 |
| Reaction time play | 20 |