Teufel Software Requirements for Supreme On Headphones

Version: 4

Referenced codes changes are located in:

Teufel Custom Commands Instructions-1.1.zip

## 1. Bluetooth Low Energy Support

1.1 As long as Bluetooth pairing is active, Bluetooth Low Energy pairing shall be active.

1.2 If Bluetooth pairing is not active, Bluetooth Low Energy pairing shall not be active.

## 2. Enable GAIA via RFComm/BLE

2.1 The GAIA interface shall be usable via RFComm over Bluetooth Classic (BR/EDR).

2.2 The GAIA interface shall be usable via GATT over Bluetooth Low Energy.

2.3 User EQ changes to bank 1 shall be persisted.

| Sink Property | Value |
| --- | --- |
| enable_gaia | ENABLE_GAIA |
| enable_gaia_rfcomm | ENABLE_GAIA_RFCOMM |
| enable_gatt | GATT_ENABLED |
| enable_gaia_persistent_user_eq_bank | ENABLE_GAIA_PERSISTENT_USER_EQ_BANK |

## 3. Enable Firmware Upgrades via GAIA

3.1 The headphone firmware shall be upgradable via GAIA.

3.2 The supplier is responsible for supplying Teufel with properly signed device firmware image files. It must be possible to upgrade launched headphones using newer versions of the device images via GAIA.

| Sink Property | Value |
| --- | --- |
| enable_upgrade | ENABLE_UPGRADE |

## 4. Custom Command Handling for "Share Me"

4.1 The sink application on the headphone should handle the following custom GAIA commands. (See proposed code changes)

| Name | GAIA Command ID | Action |
| --- | --- | --- |
| Teufel Start Share Me Pairing | 0x0800 | Starts the "Share Me" pairing procedure. |
| Teufel Stop Share Me Session | 0x0801 | Stops an active "Share Me" session. |

## 5. Custom Command Handling for "On-Ear Detection"

5.1 The sink application on the headphone should handle the following custom GAIA commands. (See proposed code changes)

| Name | GAIA Command ID | Action |
| --- | --- | --- |
| Teufel Enable On-Ear Detection | 0x0802 | Enables the "On-Ear Detection" feature. |
| Teufel Disable On-Ear Detection | 0x0803 | Disables the "On-Ear Detection" feature. |
| Teufel Get On-Ear Detection Status | 0x0804 | Returns the current state of the "On-Ear Detection" feature as payload in the GAIA acknowledgement for this command. Possible values should be 0 (DISABLED) and 1 (ENABLED). |